SOFTWARE REQUIREMENTS SPECIFICATION

for

Decentralized Edge-AI Fleet Coordination System

for Autonomous Mobile Robots in Smart Warehouses

Project ROBOTON

| Field | Value |
| --- | --- |
| Document title | SRS — Decentralized Edge-AI Fleet Coordination System |
| Document identifier | ROBOTON-SRS-001 |
| Version | 1.0 |
| Date of issue | 10 September 2026 |
| Prepared by | Team Roboton |
| Event | Smart India Hackathon 2026 |
| Problem Statement ID | 26123 |
| Problem Statement title | Edge-AI Based Distributed Fleet Coordination for Autonomous Mobile Robots (AMRs) in Smart Warehouses |
| Theme | Smart Automation |
| PS category | Software |
| Conforming to | IEEE Std 830-1998 structure, aligned to ISO/IEC/IEEE 29148:2018 |
| Status | Baselined for implementation |

This document specifies the software requirements for a fully decentralized coordination and collision-avoidance framework for a multi-robot AMR fleet operating on edge hardware. It is written to satisfy the evaluation criteria of Smart India Hackathon 2026 Problem Statement 26123; Section 8 provides the clause-by-clause requirement matching table.

# Revision History

| Version | Date | Author | Description of change |
| --- | --- | --- | --- |
| 0.1 | 30 Aug 2026 | Team Roboton | Initial idea submission; centralized planner assumed |
| 0.2 | 05 Sep 2026 | Team Roboton | Hybrid architecture: central task allocation, distributed motion coordination |
| 0.3 | 08 Sep 2026 | Team Roboton | Traffic model relocated from server to each robot; allocation moved to distributed auction |
| 0.4 | 09 Sep 2026 | Team Roboton | Insertion-cost bidding, idle preference, switching hysteresis, capacity cap added |
| 1.0 | 10 Sep 2026 | Team Roboton | Zoned auction for 100-robot scale; benchmark harness specified; baselined |

# Table of Contents

# 1. Introduction

## 1.1 Purpose

This Software Requirements Specification (SRS) defines the functional and non-functional requirements of the Decentralized Edge-AI Fleet Coordination System (hereafter “the System”), release 1.0. The System coordinates a fleet of Autonomous Mobile Robots (AMRs) in a smart warehouse without any central path planner, central task allocator, or central traffic model.

The scope of this document covers the complete System: the firmware executing on each robot, the peer-to-peer communication protocol between robots, the non-essential order gateway, the monitoring dashboard, and the simulation and benchmark harness used to demonstrate conformance to the evaluation criteria of SIH 2026 Problem Statement 26123.

## 1.2 Document Conventions

The following conventions apply throughout this document:

- “shall” denotes a mandatory requirement; “should” denotes a recommendation; “may” denotes an optional capability. This usage follows ISO/IEC/IEEE 29148:2018 §5.2.4.

- Every requirement carries a unique, permanent identifier. Identifiers are never reused once retired.

- Priority values are: Critical (evaluation criteria depend on it), High (core function), Medium (quality of result), Low (convenience).

- Numeric parameters written as tunable constants (α, ε, δ, w₁, w₂) are configuration values whose defaults are given in Appendix D and whose tuning method is specified in FR-10.10.

| Prefix | Meaning | Defined in |
| --- | --- | --- |
| ASM-n | Assumption on which the specification depends | §2.7 |
| DEP-n | External dependency | §2.8 |
| CON-n | Design or implementation constraint | §2.6 |
| IF-n | External interface requirement | §3 |
| FR-n.m | Functional requirement, grouped by feature FE-n | §4 |
| NFR-n.m | Non-functional requirement | §5 |
| BR-n | Business rule | §5.6 |
| TC-n | Verification test case | §6.2 |

## 1.3 Intended Audience and Reading Suggestions

| Reader | Recommended reading path |
| --- | --- |
| SIH evaluators / jury | §1.4 Product Scope, §2.1 Product Perspective, §8 Requirement Matching Table, §6.3 Benchmark Protocol |
| Firmware developers | §2.6 Constraints, §3.4 Communications Interfaces, §4 all features, Appendix D Memory Budget |
| Simulation & test engineers | §4.10 Metrics & Benchmark, §6 Verification, Appendix C Deadlock Proof |
| Dashboard developers | §3.1 User Interfaces, §4.8 Fleet Dashboard, §3.4 message formats |
| Faculty mentors / reviewers | §1, §2, §5 Non-functional requirements, §7 Open Issues |

## 1.4 Product Scope

The System replaces the centralized cloud or server-based path planning used in conventional warehouse AMR fleets. In conventional systems, every robot depends on a central server for route computation and task assignment, which introduces network latency proportional to fleet size, renders the fleet inoperable in Wi-Fi dead zones, and concentrates total operational risk in a single point of failure.

The System distributes all operationally necessary decision-making onto the robots themselves. Each robot learns its own model of warehouse congestion, plans its own routes, bids for its own work in a distributed auction, and negotiates right-of-way directly with its neighbours over a connectionless peer-to-peer radio protocol. An order gateway and a monitoring dashboard exist for human convenience but hold no state the fleet requires; the fleet completes all held work with both absent.

In scope:

- Peer-to-peer inter-robot messaging carrying position, intent, ETA, priority, battery and congestion data

- Per-robot learned traffic model with gossip convergence across the fleet

- Onboard route planning (A*) and incremental route repair (D* Lite) over a topological warehouse graph

- Distributed task allocation by sealed-window reverse auction with insertion-cost bidding

- Deterministic junction arbitration providing collision avoidance and provable deadlock freedom

- Exception handling for blocked aisles, robot failure, gateway loss, network loss and low battery

- A lightweight passive monitoring dashboard

- A multi-robot simulation, a stop-and-wait baseline controller, and a metrics harness sufficient to evaluate the success criteria

Out of scope for release 1.0:

- Heterogeneous fleets (differing robot speed, payload class or kinematics)

- Simultaneous Localization and Mapping (SLAM); the warehouse map is known a priori

- Automated charging-dock docking mechanics (the System routes to a charger; docking is out of scope)

- Warehouse Management System (WMS) or ERP integration beyond order ingestion

- Adversarial or Byzantine fault tolerance; all fleet members are assumed to run trusted firmware (see ASM-16, NFR-3.4)

- Multi-floor operation and lift/elevator coordination

## 1.5 Definitions, Acronyms and Abbreviations

| Term | Definition |
| --- | --- |
| AMR | Autonomous Mobile Robot — a free-navigating warehouse robot, distinct from an AGV which follows a fixed guide path. |
| Edge computing | Computation performed on the device itself rather than on a remote server. |
| ESP-NOW | Espressif's connectionless 2.4 GHz protocol allowing devices to exchange short frames directly, without an access point or an established connection. |
| Topological graph | A map representation in which vertices are junctions and edges are aisle segments carrying a traversal cost, as opposed to an occupancy grid of uniform cells. |
| A* | A best-first graph search algorithm that returns a provably least-cost path using an admissible heuristic. |
| D* Lite | An incremental replanning algorithm that repairs an existing least-cost path when edge costs change, rather than replanning from scratch. |
| Intent | A robot's declared future path: the next three junctions it will cross and its estimated time of arrival at each. |
| ETA | Estimated Time of Arrival, expressed in milliseconds from the sending robot's aligned clock. |
| Reservation | An exclusive claim by one robot on one junction for one bounded time window. |
| Safety margin | The temporal buffer (default ±800 ms) by which two reservation windows on the same junction must be separated. |
| Insertion cost | The marginal increase in a robot's committed route cost caused by adding a new task to its plan. |
| Stigmergy | Coordination achieved through traces left in a shared environment rather than direct messaging; here, digital pheromone on graph edges. |
| Pheromone | A decaying per-edge integer that rises with traversal and evaporates with time, representing live congestion. |
| EWMA | Exponentially Weighted Moving Average; here, a robot's learned estimate of the true traversal time of an edge. |
| Digest | A gossip message carrying a rotating slice of a robot's learned edge-time estimates with their sample counts. |
| Zone | A static partition of the warehouse graph used to scope task announcements at fleet scale. |
| Makespan | Elapsed time from release of the first task in a benchmark run to completion of the last. |
| Starvation | The condition in which a task is never selected because more convenient tasks always outbid it. |
| Total order | A binary relation that can rank any two elements unambiguously, with no ties and no cycles. |
| Stop-and-wait | The conventional reactive coordination baseline: a robot halts when another robot is detected within a fixed radius and resumes when the radius clears. |

## 1.6 References

| Ref | Citation |
| --- | --- |
| [R1] | IEEE Std 830-1998, IEEE Recommended Practice for Software Requirements Specifications. IEEE Computer Society, 1998. |
| [R2] | ISO/IEC/IEEE 29148:2018, Systems and software engineering — Life cycle processes — Requirements engineering. |
| [R3] | IEEE Std 1016-2009, IEEE Standard for Information Technology — Systems Design — Software Design Descriptions. |
| [R4] | Smart India Hackathon 2026, Problem Statement 26123 — Edge-AI Based Distributed Fleet Coordination for Autonomous Mobile Robots (AMRs) in Smart Warehouses. Theme: Smart Automation; Category: Software. |
| [R5] | Fourie, B., Louw, L., & Bitsch, G. (2025). Development of a Dynamic Path Planning System for Autonomous Mobile Robots Using a Multi-Agent System Approach. Sensors, 25(17), 5317. |
| [R6] | Chen, Z., Gu, T., Chen, Z., & Zhang, B. (2025). Task Allocation Algorithm and Simulation Analysis for Multiple AMRs in Digital-Intelligent Warehouses. Concurrency and Computation: Practice and Experience, 37(4–5), e8382. |
| [R7] | Keskin, M. O., Cantürk, F., Eran, C., & Aydoğan, R. (2024). Decentralised Multi-Agent Path Finding Framework and Strategies Based on Automated Negotiation. Autonomous Agents and Multi-Agent Systems, 38(1), 10. |
| [R8] | Chung, C. H., & Jang, Y. J. (2024). Deadlock Prevention and Multi-Agent Path Finding Algorithm Considering Physical Constraints for a Massive Fleet AGV System. Applied Soft Computing, 161, 111725. |
| [R9] | Smith, R. G. (1980). The Contract Net Protocol: High-Level Communication and Control in a Distributed Problem Solver. IEEE Transactions on Computers, C-29(12), 1104–1113. |
| [R10] | Koenig, S., & Likhachev, M. (2002). D* Lite. Proceedings of the AAAI Conference on Artificial Intelligence, 476–483. |
| [R11] | Hart, P. E., Nilsson, N. J., & Raphael, B. (1968). A Formal Basis for the Heuristic Determination of Minimum Cost Paths. IEEE Transactions on Systems Science and Cybernetics, 4(2), 100–107. |
| [R12] | Espressif Systems. ESP-NOW User Guide and ESP32 Technical Reference Manual. |

# 2. Overall Description

## 2.1 Product Perspective

The System is a self-contained replacement for the coordination layer of a warehouse AMR fleet. It is not a component of a larger product and does not require a Warehouse Management System to function, although one may supply orders through the gateway interface (IF-3).

### 2.1.1 Architectural partitioning rule

The System is partitioned by a single rule, from which all other architectural decisions follow:

If the fleet cannot complete its work without a component, that component executes on the robots.

This yields two strata. Every element of the lower stratum is replicated identically on every robot; no element of the upper stratum holds state that the lower stratum requires.

| Stratum | Components | Deadline | Loss behaviour |
| --- | --- | --- | --- |
| Robot (essential) | Traffic model; A*/D* Lite planner; auction bidder; reservation table; junction arbitration; motion control; localization | 5–50 ms | Fleet cannot operate; equivalent to losing the robot |
| Off-robot (optional) | Order gateway; monitoring dashboard | seconds | Held tasks complete normally; only new order ingestion and monitoring pause |

### 2.1.2 System context

The context of the System comprises four external entities:

- The Warehouse Operator, who introduces orders through the gateway.

- The Fleet Dashboard, a passive listener that renders fleet state and issues no commands.

- The physical warehouse, represented internally as a static topological graph with dynamic edge blockages.

- Non-cooperative obstacles — humans, pallets, dropped goods — which are detected by onboard sensors but do not participate in the coordination protocol.

## 2.2 Product Functions

At the highest level the System provides ten functions, each specified as a feature in §4:

| Feature | Function |
| --- | --- |
| FE-1 | Decentralized peer-to-peer communication of position, intent, ETA and state |
| FE-2 | Per-robot learned traffic model with gossip convergence |
| FE-3 | Onboard route planning and incremental route repair |
| FE-4 | Distributed task allocation by reverse auction |
| FE-5 | Deterministic junction arbitration and collision avoidance |
| FE-6 | Exception handling, re-routing and task re-assignment |
| FE-7 | Localization and fleet clock alignment |
| FE-8 | Passive fleet monitoring dashboard |
| FE-9 | Zoned scalability to large fleets |
| FE-10 | Metrics capture, baseline controller and benchmark harness |

## 2.3 User Classes and Characteristics

| User class | Characteristics | Interaction with the System | Frequency |
| --- | --- | --- | --- |
| Warehouse Operator | Non-technical; trained on order entry only | Enters pickup/drop orders with a priority value through the gateway UI | Continuous during shift |
| Fleet / Floor Manager | Operationally experienced; not a roboticist | Monitors the dashboard; escalates alerts; does not direct individual robots | Continuous, supervisory |
| Warehouse Safety Personnel | Trained on floor safety procedure | Observes collision and fault indicators; operates physical emergency stop | On exception |
| Maintenance Technician | Technically trained on the robot hardware | Replaces batteries, re-seats floor markers, reflashes firmware | Scheduled and on fault |
| Developer / Evaluator | Software and robotics literate | Runs the simulation, the baseline controller and the benchmark harness; reads logged metrics | During development and evaluation |

## 2.4 Operating Environment

| Element | Specification |
| --- | --- |
| Robot compute | ESP32 (dual-core Xtensa LX6, 240 MHz, ~520 KB SRAM of which ~320 KB usable, no MMU, no operating system beyond FreeRTOS) |
| Robot radio | Integrated 2.4 GHz; ESP-NOW for peer traffic, station-mode Wi-Fi for optional gateway traffic |
| Robot drive | Differential drive, two driven wheels with quadrature encoders, supplementary IMU |
| Robot sensing | Forward obstacle sensor; downward floor-marker reader (IR, RFID or fiducial); battery voltage via ADC |
| Gateway | Raspberry Pi class single-board computer or laptop, running the order ingestion service and the ESP-NOW bridge |
| Dashboard | Modern web browser on any device on the same network; no plug-ins |
| Simulation | Webots or Gazebo on a Linux or Windows workstation, hosting ≥3 AMR models on the benchmark map |
| Physical environment | Flat indoor warehouse floor; static aisle topology; junctions instrumented with ground markers; ambient 2.4 GHz congestion typical of an industrial site |

## 2.5 Design Rationale for Key Decisions

Three decisions in this specification depart from the conventional approach and are recorded here with their justification, because they constrain many downstream requirements.

| Decision | Alternative rejected | Rationale |
| --- | --- | --- |
| Topological graph, not an occupancy grid | Uniform grid map with grid-based A* | A 500×500 grid holds 250,000 cells; storing cost, g and rhs values per cell exceeds ESP32 SRAM by orders of magnitude. A warehouse floor plan is known in advance, so junctions and aisle segments are the natural and far smaller representation. See Appendix D. |
| Deterministic arbitration, no stochastic optimizer in the safety path | Particle swarm or ant colony optimization for conflict resolution | Two robots running independent stochastic solvers on the same conflict can both conclude they hold right of way. Determinism guarantees identical conclusions fleet-wide and makes defects reproducible. Bio-inspired methods are used only where a non-deterministic answer is tolerable (FR-2.3, FR-10.10). |
| Distributed auction, not central allocation | Greedy assignment on a server | Once the traffic model is per-robot (FR-2.1), only the robot holds the cost information needed to assign work correctly. A central allocator would bid on the robot's behalf using data it no longer possesses, and would reinstate the single point of failure the System exists to remove. |

## 2.6 Design and Implementation Constraints

| ID | Constraint | Consequence |
| --- | --- | --- |
| CON-1 | ESP32 provides ~320 KB usable SRAM, no MMU and no virtual memory | Map, traffic model and planner working set must fit the budget in Appendix D; dynamic allocation is prohibited in the steady-state loop |
| CON-2 | ESP-NOW frames carry a maximum payload of approximately 250 bytes | All coordination messages must fit one frame; DIGEST must send a rotating slice rather than a whole table (FR-2.5) |
| CON-3 | ESP-NOW unicast peer tables are capacity limited | Coordination uses broadcast rather than per-peer unicast, which also removes any peer-count ceiling |
| CON-4 | The 2.4 GHz band is a shared medium; airtime contention degrades worse than linearly with the number of transmitters | Fleet-wide flood auctions are not viable at scale; announcements must be zone-scoped (FE-9) |
| CON-5 | No LiDAR and insufficient compute for SLAM | Localization is odometry plus ground-marker re-zeroing (FE-7); position confidence must be tracked explicitly |
| CON-6 | ESP32 floating-point performance is limited | Cost arithmetic shall use fixed-point or integer representations in the 5 ms arbitration path |
| CON-7 | Safety decisions must be reproducible for testing and certification | No randomized tie-break, no stochastic search and no learned model may participate in the arbitration path (FR-2.9, FR-5.9) |
| CON-8 | The problem statement names Raspberry Pi or Jetson Nano as reference edge hardware | The planner shall remain portable and buildable for a Linux single-board computer without source changes (NFR-4.3); the deviation is justified in §8.2 |
| CON-9 | The mandated deliverable is a multi-robot simulation with at least three AMRs | The simulation, baseline and metrics harness are requirements of equal standing to the firmware (FE-10) |
| CON-10 | Node and edge identifiers are transmitted as 8-bit fields | The warehouse graph shall not exceed 255 nodes; larger sites require a field-width revision |

## 2.7 Assumptions

The specification is valid only while the following assumptions hold. Each carries the consequence of its violation, so that a reviewer can judge the exposure.

| ID | Assumption | If violated |
| --- | --- | --- |
| ASM-1 | The warehouse aisle topology is known in advance and is static; only edge traversability changes at run time | Requires SLAM and dynamic graph construction — outside release 1.0 |
| ASM-2 | The warehouse graph contains at most 255 nodes and 255 edges addressable in 8-bit fields | Packet field widths and memory budget must be revised (CON-10) |
| ASM-3 | A machine-readable ground marker is installed at every junction | Odometry drift accumulates unbounded; ETAs become unreliable and every reservation built on them is invalid |
| ASM-4 | Robots are differential-drive and operate on a flat, dry, indoor floor | Kinematic and traction models in the motion controller no longer apply |
| ASM-5 | Each aisle segment is labelled in the map as single-lane or bidirectional | Head-on conflicts in single-lane corridors cannot be anticipated; FR-5.10 cannot be satisfied |
| ASM-6 | All robots are homogeneous in maximum speed, acceleration and payload class | Bid comparability breaks down; insertion costs are no longer expressed in a common unit |
| ASM-7 | Any two robots that can physically collide are within ESP-NOW radio range of one another | The safety guarantee is void. This is the single most critical assumption in the document; it is verified by TC-26 |
| ASM-8 | Radio packet loss over one 200 ms intent period is intermittent rather than sustained | Reservation tables go stale; the System falls back to conservative behaviour under FR-5.13 but throughput drops |
| ASM-9 | Clock drift between ESP32 oscillators is bounded such that a 5 s beacon holds offsets within the safety margin | Reservation windows become incomparable; FR-7.6 widens the margin as mitigation |
| ASM-10 | Every task pickup and drop location corresponds to a node in the graph | Requires last-metre navigation off-graph — outside release 1.0 |
| ASM-11 | The order gateway may be absent at any time and for any duration | This is a design assumption, not a risk; the System is required to tolerate it (FR-6.3) |
| ASM-12 | Battery state of charge is measurable to within ±5% | Battery penalty terms misprice bids; a robot may accept work it cannot finish |
| ASM-13 | Humans and non-fleet obstacles are detected by onboard sensors but do not broadcast intent | Non-cooperative obstacles are handled reactively by FR-6.6, at reduced throughput |
| ASM-14 | The graded SIH deliverable is the simulation; physical robots are a supporting demonstrator | If physical hardware is graded, the ESP32 platform choice must be re-argued against CON-8 |
| ASM-15 | Robots run identical, trusted firmware; no adversarial participant is present | Message authenticity cannot be assumed; NFR-3 mitigations become insufficient |
| ASM-16 | Zone membership is assigned statically at boot from the map | Dynamic zone rebalancing would be required, adding consensus cost not budgeted here |
| ASM-17 | Task priority is supplied by the operator as an integer in the range 0–255 | Priority comparison in the arbitration total order is undefined |
| ASM-18 | At most two tasks are queued per robot (capacity cap) | Insertion-cost computation grows combinatorially with queue length |

## 2.8 Dependencies

| ID | Dependency | Nature |
| --- | --- | --- |
| DEP-1 | Espressif ESP-IDF or Arduino-ESP32 with the ESP-NOW API | Build and runtime dependency of the firmware |
| DEP-2 | FreeRTOS task scheduler as shipped with ESP-IDF | Runtime dependency for the concurrent broadcast, planning and motion loops |
| DEP-3 | Webots or Gazebo robot simulator | Required to produce the mandated deliverable and to execute the benchmark |
| DEP-4 | A browser-based charting and rendering stack for the dashboard | Presentation only; no fleet function depends on it |
| DEP-5 | Availability of ground markers and their installation at junctions | Physical deployment dependency; simulated in the graded deliverable |

# 3. External Interface Requirements

## 3.1 User Interfaces

| ID | Requirement | Pri. |
| --- | --- | --- |
| IF-1.1 | The dashboard shall render the live position of every AMR on a plan view of the warehouse graph. | Critical |
| IF-1.2 | The dashboard shall display, for every AMR, its battery state of charge as a percentage and as a graded visual indicator. | Critical |
| IF-1.3 | The dashboard shall display, for every AMR, its current state, current task identifier and task progress. | High |
| IF-1.4 | The dashboard shall display active alerts: robot faults, unreachable tasks, blocked edges, low-battery warnings and any detected collision. | High |
| IF-1.5 | The dashboard shall present a fleet health summary comprising robot count by state, task queue depth and gateway connectivity status. | Medium |
| IF-1.6 | The dashboard shall provide no control affordance capable of commanding a robot; it is strictly read-only. | Critical |
| IF-1.7 | The dashboard shall indicate visually when its own data feed is stale or disconnected, and shall not present stale data as current. | High |
| IF-1.8 | The gateway order entry interface shall accept a pickup node, a drop node and a priority in the range 0–255, and shall reject nodes not present in the map. | High |

## 3.2 Hardware Interfaces

| ID | Interface | Requirement |
| --- | --- | --- |
| IF-2.1 | Wheel encoders | The firmware shall read quadrature encoder counts at not less than 50 Hz for odometric position estimation. |
| IF-2.2 | Inertial measurement unit | The firmware shall fuse IMU heading with encoder odometry to bound heading drift between markers. |
| IF-2.3 | Ground marker reader | The firmware shall decode a junction identifier from the floor marker and shall re-zero the position estimate on every successful decode. |
| IF-2.4 | Obstacle sensor | The firmware shall detect a non-cooperative obstacle ahead within the stopping distance at maximum commanded speed. |
| IF-2.5 | Motor drivers | The firmware shall issue differential velocity commands at not less than 50 Hz. |
| IF-2.6 | Battery monitor | The firmware shall sample battery voltage and derive state of charge at not less than 1 Hz. |
| IF-2.7 | Emergency stop | A hardware emergency stop shall cut motor power independently of the firmware and shall not require software cooperation to take effect. |

## 3.3 Software Interfaces

| ID | Interface | Requirement |
| --- | --- | --- |
| IF-3.1 | Simulator interface | The coordination logic shall be compiled and executed unchanged against the simulator's robot model, so that simulated and physical robots run identical decision code. |
| IF-3.2 | Gateway ingestion | The gateway shall accept orders and emit them onto the mesh as ANNOUNCE messages without evaluating, ranking or assigning them. |
| IF-3.3 | Gateway bridge | The gateway shall relay overheard mesh traffic to the dashboard transport and shall not originate any message other than ANNOUNCE and the clock beacon. |
| IF-3.4 | Metrics export | The benchmark harness shall export per-run metrics in a machine-readable delimited format suitable for statistical aggregation. |

## 3.4 Communications Interfaces

All coordination traffic uses ESP-NOW broadcast frames. Five message types constitute the complete protocol.

### 3.4.1 INTENT

Broadcast by every robot every 200 ms. This is the only message on the continuous safety-critical path and feeds both the reservation table and the traffic model.

| Field | Type | Purpose |
| --- | --- | --- |
| robot_id | uint8 | Identity of the sender; also the arbitration tie-break key |
| seq | uint16 | Monotonic sequence number for de-duplication and loss detection |
| current_node | uint8 | Junction most recently occupied or departed |
| next_nodes[3] | uint8[3] | The next three junctions the sender intends to cross |
| eta_ms[3] | uint16[3] | Estimated arrival at each of those junctions, in ms from the sender's aligned clock |
| priority | uint8 | Composite of task urgency, battery state and task aging, precomputed by the sender |
| state | uint8 | MOVING, YIELD, REPLAN, FAULT or CHARGING |
| battery_pct | uint8 | State of charge; consumed by the dashboard and by peers' bid evaluation |
| pheromone | uint8 | Congestion deposit for the edge just traversed |
| timestamp | uint32 | Sender's aligned clock at transmission, for reservation window arithmetic |
| crc | uint16 | Integrity check; malformed frames are discarded |

### 3.4.2 Remaining message types

| Message | Direction | Rate | Safety-critical | Payload summary |
| --- | --- | --- | --- | --- |
| RESERVE | robot ↔ robot | on claim | Yes | junction_id, window_start_ms, window_end_ms, priority, robot_id |
| ANNOUNCE | gateway → mesh | per task | Yes | task_id, pickup_node, drop_node, priority, created_at, zone_id |
| BID / CLAIM | robot ↔ robot | per task | Yes | task_id, bid_value, robot_id, is_idle flag; CLAIM asserts the win |
| DIGEST | robot ↔ robot | 0.1 Hz | No | Rotating slice of edge_id, ewma_time, sample_count triples |
| TELEMETRY | mesh → dashboard | 1 Hz | No | Overheard state relayed by the gateway; the dashboard subscribes and never polls |

| ID | Requirement | Pri. |
| --- | --- | --- |
| IF-4.1 | Every coordination message shall fit within a single ESP-NOW frame of at most 250 bytes of payload. | Critical |
| IF-4.2 | INTENT and RESERVE shall not traverse any access point, router or server. | Critical |
| IF-4.3 | Every message shall carry an integrity check; frames failing it shall be discarded without effect. | High |
| IF-4.4 | Every message shall carry a sequence number and timestamp sufficient to reject replayed or out-of-order frames. | High |
| IF-4.5 | No message type on the safety-critical path shall require acknowledgement, retransmission or an established connection. | Critical |

# 4. System Features

Each feature below is stated with its description, its stimulus/response behaviour, and its associated functional requirements. Requirements are testable and are traced to verification cases in §7.2.

## 4.1 FE-1 — Decentralized Peer Communication

Description: Robots exchange position, intent, ETA, priority, battery and congestion data directly with one another over a connectionless peer-to-peer protocol, with no server, access point or broker participating.

Stimulus / response: On each 200 ms tick a robot broadcasts INTENT. On receipt of a peer's INTENT, a receiving robot updates its reservation table and its pheromone counters from the same frame.

| ID | Requirement | Pri. |
| --- | --- | --- |
| FR-1.1 | Each AMR shall broadcast an INTENT message every 200 ms with a tolerance of ±20 ms. | Critical |
| FR-1.2 | The INTENT message shall carry every field specified in §3.4.1. | Critical |
| FR-1.3 | Exchange of INTENT and RESERVE shall require no central server, access point or broker. | Critical |
| FR-1.4 | Each AMR shall maintain a peer table of neighbours from which INTENT has been received within the liveness window. | High |
| FR-1.5 | Each AMR shall declare a peer lost when no INTENT has been received from it for more than 1500 ms. | Critical |
| FR-1.6 | Each AMR shall discard a received frame whose sequence number is not greater than the last accepted sequence number from that sender. | High |
| FR-1.7 | The System shall continue full peer coordination when the Wi-Fi access point is unavailable. | Critical |
| FR-1.8 | A single received INTENT frame shall update both the reservation table and the traffic model, without a second message being required. | Medium |

## 4.2 FE-2 — Per-Robot Traffic Model and Learning

Description: Each robot maintains its own model of warehouse congestion, learned from what it overhears and what it experiences. No central copy of the model exists and no robot is authoritative for it. The model operates at three timescales which collapse into a single edge cost consumed by the planner.

Stimulus / response: On traversing an edge, the robot updates its learned traversal time for that edge and deposits pheromone. On receiving a DIGEST, it merges peer estimates weighted by sample count. Every 2 s pheromone evaporates.

| Layer | Horizon | Source | Used as |
| --- | --- | --- | --- |
| Reservations | 0–3 s | Peers' INTENT: their next three junctions and ETAs | Hard constraint; never a cost term |
| Pheromone | 5–60 s | Own traversals and the deposit byte in every overheard INTENT | Soft additive cost; decays to zero |
| Learned edge time | minutes–shift | EWMA of own measured traversal times, merged with peers' DIGEST gossip | Base cost, replacing the nominal map cost |

edge_cost(e) = ewma_time[e]                 // learned base cost

+ alpha * pheromone[e]         // live congestion, decays every 2 s

+ INFINITY if reserved_conflict(e)   // hard block from peers' intent

| ID | Requirement | Pri. |
| --- | --- | --- |
| FR-2.1 | Each AMR shall maintain its traffic model locally; the System shall contain no central traffic model. | Critical |
| FR-2.2 | The traffic model shall comprise the three layers of §4.2 with the stated horizons. | Critical |
| FR-2.3 | Pheromone values shall decay towards zero on a period of 2000 ms. | High |
| FR-2.4 | On completing traversal of an edge, an AMR shall update its exponentially weighted moving average of that edge's traversal time. | High |
| FR-2.5 | Each AMR shall broadcast a DIGEST every 10 s carrying a rotating slice of its learned edge estimates with their sample counts, sized to fit one frame. | High |
| FR-2.6 | On receiving a DIGEST, an AMR shall merge each entry into its own estimate weighted by the relative sample counts. | High |
| FR-2.7 | An AMR joining an operating fleet shall initialise its model to the nominal map costs and shall converge towards the fleet estimate without operator intervention. | Medium |
| FR-2.8 | The complete traffic model shall occupy not more than 1 KB of SRAM. | Critical |
| FR-2.9 | The traffic model shall not be consulted by, and shall have no influence upon, the junction arbitration path of FE-5. | Critical |
| FR-2.10 | Divergence between the models held by different AMRs shall not compromise any safety property; it shall affect route selection only. | Critical |

Note on divergence: robots will not hold identical cost tables and will therefore not always select identical routes. This is safe because collision avoidance depends solely on the reservation layer, which is driven by broadcast facts rather than learned estimates (FR-2.9). It is also beneficial: identical models would direct identical robots along identical least-cost aisles and manufacture the congestion they were computed to avoid.

## 4.3 FE-3 — Route Planning and Incremental Repair

Description: Each robot computes its own route over a topological graph using A*, and repairs that route incrementally with D* Lite when edge costs change or an edge becomes impassable.

Stimulus / response: On winning a task the robot plans a route to pickup and onward to drop. On learning that an edge is impassable it repairs only the affected portion of the existing route.

| ID | Requirement | Pri. |
| --- | --- | --- |
| FR-3.1 | The warehouse shall be represented onboard as a topological graph whose vertices are junctions and whose edges are aisle segments with traversal costs. | Critical |
| FR-3.2 | Each AMR shall compute its initial route using A* with an admissible heuristic. | Critical |
| FR-3.3 | Each AMR shall repair an invalidated route using D* Lite, replanning only the affected portion rather than the whole route. | Critical |
| FR-3.4 | The planner shall consume the composed edge cost of FR-2.2 without special-case handling of its constituent layers. | High |
| FR-3.5 | Initial planning shall complete within 50 ms for any graph conforming to ASM-2. | Critical |
| FR-3.6 | Route repair shall complete within 50 ms. | Critical |
| FR-3.7 | Where no traversable route to a task exists, the AMR shall declare the task UNREACHABLE and return it to the mesh under FR-6.2. | High |
| FR-3.8 | The planner shall use fixed-point or integer arithmetic in any path executed within the arbitration deadline. | Medium |
| FR-3.9 | A route shall not be planned through an edge currently subject to a conflicting reservation. | Critical |

## 4.4 FE-4 — Distributed Task Allocation by Auction

Description: Tasks are not assigned. A task is announced to the mesh, each eligible robot computes what that task would cost it, and the lowest bidder takes it. The mechanism is a sealed-window reverse auction in the tradition of the Contract Net Protocol [R9], with the bid expressed as the marginal cost of inserting the task into the bidder's existing plan.

Stimulus / response: On receiving ANNOUNCE for a task in its zone or an adjacent zone, an eligible robot computes and broadcasts a BID. When the 300 ms window closes, every robot independently sorts the bids it heard and reaches the same conclusion; the winner broadcasts CLAIM.

insertion_cost = cost(committed route WITH task inserted at best point)

- cost(committed route unchanged)

bid = insertion_cost

+ w1 * battery_penalty        // low charge raises the bid

- w2 * task_waiting_time      // aging: starvation prevention

- epsilon * is_idle           // small preference on a near tie

Insertion cost, rather than raw distance, is what makes the choice between an idle robot and a busy one resolve correctly without a hand-written rule. An idle robot bids its full travel cost because it has nothing to disrupt. A busy robot bids only the detour the task adds to its committed plan: small where the task lies on its way, large where it would have to backtrack. A raw-distance bid would let a busy robot near the pickup outbid every idle robot while silently charging the delay to the task it had already promised.

| ID | Requirement | Pri. |
| --- | --- | --- |
| FR-4.1 | Tasks shall be introduced to the fleet by broadcast ANNOUNCE; no component shall assign a task to a named robot. | Critical |
| FR-4.2 | Each eligible AMR shall compute its own bid using its own map, its own traffic model and its own committed route. | Critical |
| FR-4.3 | The bid shall be the marginal insertion cost of the task into the bidder's committed plan, adjusted by the battery, aging and idle terms of §4.4. | Critical |
| FR-4.4 | The bidding window shall be 300 ms measured from the ANNOUNCE timestamp. | High |
| FR-4.5 | The winner shall be the lowest bid; equal bids shall be resolved in favour of the lower robot_id. | Critical |
| FR-4.6 | Each AMR shall determine the winner independently from the bids it received; no dispatcher shall exist and no robot shall be informed of the outcome. | Critical |
| FR-4.7 | The winning AMR shall broadcast CLAIM before transitioning to PLANNING. | Critical |
| FR-4.8 | Where two AMRs broadcast CLAIM for the same task, the lower robot_id shall retain it and the other shall relinquish it within one auction cycle. | Critical |
| FR-4.9 | An AMR holding the maximum queue of two tasks shall not bid. | High |
| FR-4.10 | A task already claimed shall transfer to a challenger only where the challenger's bid is lower than the holder's by more than the stability margin δ. | High |
| FR-4.11 | The aging term shall guarantee that no task remains unallocated indefinitely while any AMR is capable of performing it. | Critical |
| FR-4.12 | Where the winner does not broadcast CLAIM within 2000 ms, the second-place bidder shall re-announce the task. | High |
| FR-4.13 | Where no bid is received for an announced task, the task shall be re-announced with its aging term continuing to accrue until it is claimed. | High |
| FR-4.14 | Only AMRs registered in the task's zone or in a zone adjacent to it shall bid. | High |
| FR-4.15 | An AMR whose battery is below the reserve threshold shall not bid. | High |

## 4.5 FE-5 — Junction Arbitration and Collision Avoidance

Description: Conflicts are resolved in time rather than in space. Two robots do not conflict because they use the same junction, but because they would use it within the same time window. Each robot maintains a local reservation table built from peers' broadcast intent and arbitrates entry deterministically.

Stimulus / response: Before committing to a junction the robot detects any overlapping claim within the safety margin, compares priority, and either reserves the junction or yields by shedding speed or selecting an alternate edge.

| ID | Requirement | Pri. |
| --- | --- | --- |
| FR-5.1 | Each AMR shall maintain a local reservation table of claimed junction time windows derived from received INTENT and RESERVE messages. | Critical |
| FR-5.2 | Before committing to a junction, an AMR shall execute the sequence Detect, Prioritize, Reserve, Avoid. | Critical |
| FR-5.3 | Two reservation windows on the same junction shall be treated as conflicting where they are separated by less than the safety margin, whose default value is 800 ms. | Critical |
| FR-5.4 | Arbitration shall rank contenders by the total order (priority descending, robot_id ascending). | Critical |
| FR-5.5 | The winning AMR shall broadcast RESERVE for its window before entering the junction. | Critical |
| FR-5.6 | A yielding AMR shall resolve the conflict by reducing speed to shift its ETA beyond the conflicting window, or by selecting an alternate edge; it shall not resolve it by emergency braking. | High |
| FR-5.7 | Arbitration shall reach a decision within 5 ms of detecting the conflict. | Critical |
| FR-5.8 | Arbitration shall be deterministic: identical reservation-table contents shall produce identical decisions on every AMR. | Critical |
| FR-5.9 | No randomized, stochastic or learned component shall participate in arbitration. | Critical |
| FR-5.10 | In a single-lane corridor an AMR shall complete its yield decision at the last passing point preceding the corridor, before committing to entry. | Critical |
| FR-5.11 | Reservations shall expire automatically at the end of their window without requiring a release message. | High |
| FR-5.12 | The System shall be free of cyclic deadlock by construction; the proof obligation is discharged in Appendix C. | Critical |
| FR-5.13 | Where the clock beacon has not been received for more than 60 s, the safety margin shall be widened to 1500 ms. | High |
| FR-5.14 | Where an AMR's position confidence is below the configured threshold, it shall not claim a junction reservation until confidence is restored by a marker read. | Critical |

## 4.6 FE-6 — Exception Handling, Re-routing and Re-assignment

Description: Every failure is handled locally first and escalated to the mesh second; no failure is escalated to a server, because none exists. The System defines graduated degradation levels rather than a binary operating/failed distinction.

| ID | Requirement | Pri. |
| --- | --- | --- |
| FR-6.1 | On detecting a blocked aisle, an AMR shall mark the edge impassable, repair its route under FR-3.3, and broadcast the blockage so that peers avoid the edge. | Critical |
| FR-6.2 | On declaring a task UNREACHABLE, an AMR shall re-announce it to the mesh and return to IDLE. | High |
| FR-6.3 | On loss of the order gateway, AMRs shall complete all held tasks, continue arbitration unchanged, and buffer telemetry locally. | Critical |
| FR-6.4 | On loss of the Wi-Fi access point, coordination shall be unaffected. | Critical |
| FR-6.5 | On declaring a peer lost, an AMR shall expire that peer's reservations, treat its last known node as an obstacle, and re-announce any task it is known to have held. | Critical |
| FR-6.6 | On detecting a non-cooperative obstacle, an AMR shall stop safely, wait for a configured interval, and then repair its route around the obstruction. | Critical |
| FR-6.7 | On battery falling below the reserve threshold, an AMR shall cease bidding, complete its current task, and route to a charging node. | High |
| FR-6.8 | On position confidence falling below threshold, an AMR shall reduce speed and navigate to the nearest ground marker to re-establish position. | Critical |
| FR-6.9 | The System shall define and report its degradation level: NORMAL, DEGRADED (gateway or dashboard unavailable) or RESTRICTED (clock or localization confidence reduced). | Medium |

## 4.7 FE-7 — Localization and Clock Alignment

Description: Coordination is expressed in time windows, so both position and time must be trustworthy. Position is maintained by odometry between junctions and re-zeroed at ground markers; clocks are aligned by a periodic beacon.

This feature is load-bearing for FE-5. Where position drifts, ETAs are wrong, and every reservation computed from them is fiction. Ground markers make position trustworthy at precisely the locations where coordination occurs.

| ID | Requirement | Pri. |
| --- | --- | --- |
| FR-7.1 | Each AMR shall estimate its position between junctions from wheel encoders fused with IMU heading. | Critical |
| FR-7.2 | Each AMR shall re-zero its position estimate on every successful ground-marker decode. | Critical |
| FR-7.3 | Each AMR shall maintain an explicit confidence value for its position estimate, degrading with distance travelled since the last marker. | Critical |
| FR-7.4 | A clock beacon shall be broadcast every 5 s; any AMR or the gateway may originate it. | High |
| FR-7.5 | Each AMR shall maintain a local offset to the fleet-aligned clock and shall express all ETAs and reservation windows on that aligned clock. | Critical |
| FR-7.6 | Where beacon age exceeds the configured bound, the AMR shall widen its safety margin under FR-5.13 rather than continue at nominal margin. | Critical |

## 4.8 FE-8 — Fleet Dashboard

Description: A lightweight monitoring surface that renders fleet state by subscribing to relayed mesh traffic. It observes and never commands.

| ID | Requirement | Pri. |
| --- | --- | --- |
| FR-8.1 | The dashboard shall display the real-time position of every AMR in the fleet. | Critical |
| FR-8.2 | The dashboard shall display the battery status of every AMR in the fleet. | Critical |
| FR-8.3 | The dashboard shall refresh at not less than 1 Hz. | High |
| FR-8.4 | The dashboard shall be passive; it shall issue no command capable of altering AMR behaviour. | Critical |
| FR-8.5 | The fleet shall operate at full capability while the dashboard is disconnected or absent. | Critical |
| FR-8.6 | The dashboard shall derive robot state from data already present in INTENT, requiring no additional query path to the robots. | Medium |
| FR-8.7 | The dashboard shall run in a standard web browser without plug-ins and without a heavyweight server component. | High |

## 4.9 FE-9 — Zoned Scalability

Description: The architecture is unchanged at fleet scale; what changes is scope. A flood auction costs approximately 2N frames per task for a fleet of N robots, and because the 2.4 GHz band is a shared medium (CON-4), contention degrades worse than linearly. Announcements are therefore scoped to the task's zone and its adjacent zones, while INTENT and RESERVE remain unscoped because their locality is already physical.

| Fleet size | Flood auction | Zoned auction | Assessment |
| --- | --- | --- | --- |
| 3–5 AMRs | 6–10 frames per task | Not required | Zoning adds complexity without benefit; flood is correct |
| ~20 AMRs | ~40 frames per task | ~14 frames per task | Zoning becomes worthwhile |
| 100 AMRs | ~200 frames per task; bids lost to contention | ~34 frames per task | Zoning required; flood degrades silently |

| ID | Requirement | Pri. |
| --- | --- | --- |
| FR-9.1 | The warehouse graph shall be partitioned into zones, assigned to AMRs at boot from the map. | High |
| FR-9.2 | ANNOUNCE shall carry the zone identifier of the task. | High |
| FR-9.3 | Only AMRs in the task's zone or an adjacent zone shall be eligible to bid. | High |
| FR-9.4 | INTENT and RESERVE shall not be zone-scoped; their scope is the physical radio neighbourhood, which is the set of robots capable of colliding. | Critical |
| FR-9.5 | The System shall support a minimum of three AMRs and shall be designed to accommodate one hundred without architectural change. | Critical |
| FR-9.6 | Auction traffic per task shall scale with fleet size divided by zone count, not with fleet size. | High |
| FR-9.7 | Zone assignment shall not introduce any component whose loss prevents the fleet from operating. | Critical |

## 4.10 FE-10 — Metrics, Baseline Controller and Benchmark Harness

Description: The evaluation criteria of the problem statement are measurements, not assertions. A claim of zero collisions requires a collision detector; a claim of a twenty per cent improvement requires a baseline against which to measure. Both are specified here as requirements of equal standing to the coordination firmware itself.

| ID | Requirement | Pri. |
| --- | --- | --- |
| FR-10.1 | The harness shall detect and log every inter-robot collision event with timestamp and participants. | Critical |
| FR-10.2 | The harness shall log per-task completion time and the makespan of each run. | Critical |
| FR-10.3 | The harness shall log total time each AMR spends stopped and the count of yields granted and received. | High |
| FR-10.4 | The System shall include a stop-and-wait baseline controller implementing conventional reactive coordination. | Critical |
| FR-10.5 | The baseline shall use round-robin task assignment, shall exchange no intent, and shall halt an AMR when another is within a fixed radius, resuming when that radius clears. | Critical |
| FR-10.6 | The baseline and the System shall be executed on an identical map, an identical task set and identical random seeds. | Critical |
| FR-10.7 | Each configuration shall be executed for not fewer than ten seeds and reported as mean with standard deviation. | Critical |
| FR-10.8 | The benchmark map shall contain at least one single-lane choke corridor and at least two longer alternative routes bypassing it. | Critical |
| FR-10.9 | The benchmark task set shall be constructed such that not less than sixty per cent of tasks induce a route through the choke corridor, producing the overlapping paths the evaluation criteria require. | Critical |
| FR-10.10 | The tunable constants α, ε, δ, w₁ and w₂ shall be determined by offline search in simulation against the makespan objective, and shipped as fixed constants; no such search shall execute at run time. | Medium |

# 5. Other Non-functional Requirements

## 5.1 Performance Requirements

| ID | Requirement | Value |
| --- | --- | --- |
| NFR-1.1 | Junction arbitration decision latency | ≤ 5 ms |
| NFR-1.2 | Initial route planning latency | ≤ 50 ms |
| NFR-1.3 | Route repair latency | ≤ 50 ms |
| NFR-1.4 | INTENT broadcast rate | 5 Hz ±10% |
| NFR-1.5 | Motion control loop rate | ≥ 50 Hz |
| NFR-1.6 | DIGEST gossip rate | 0.1 Hz |
| NFR-1.7 | Telemetry relay rate | 1 Hz |
| NFR-1.8 | Auction window | 300 ms |
| NFR-1.9 | Traffic model memory footprint | ≤ 1 KB |
| NFR-1.10 | Total application SRAM footprint | ≤ 40 KB (see Appendix D) |
| NFR-1.11 | Peer loss detection latency | ≤ 1500 ms |
| NFR-1.12 | Auction frames per task at 100 AMRs across 6 zones | ≤ 40 |

## 5.2 Safety Requirements

| ID | Requirement | Pri. |
| --- | --- | --- |
| NFR-2.1 | The System shall produce zero inter-robot collisions across the full benchmark suite. | Critical |
| NFR-2.2 | The System shall be free of cyclic deadlock by construction rather than by detection and recovery. | Critical |
| NFR-2.3 | An AMR that cannot establish its position with sufficient confidence shall not enter a junction. | Critical |
| NFR-2.4 | Coordination shall resolve conflicts by anticipation; emergency braking shall be reserved for non-cooperative obstacles and shall not be the normal means of conflict resolution. | High |
| NFR-2.5 | Every decision in the safety path shall be deterministic and reproducible from logged inputs. | Critical |
| NFR-2.6 | The hardware emergency stop shall function independently of the coordination software. | Critical |
| NFR-2.7 | Loss of any single AMR, of the gateway, of the dashboard or of the access point shall not create a collision hazard for the remaining fleet. | Critical |

## 5.3 Security Requirements

| ID | Requirement | Pri. |
| --- | --- | --- |
| NFR-3.1 | Coordination frames shall carry an integrity check sufficient to detect corruption, and corrupted frames shall be discarded without effect. | High |
| NFR-3.2 | Frames shall be protected against replay by sequence number and timestamp validation. | High |
| NFR-3.3 | Frames whose field values fall outside the valid ranges defined in §3.4 shall be discarded without effect. | High |
| NFR-3.4 | Release 1.0 assumes a trusted fleet (ASM-15). Cryptographic peer authentication is identified as a known limitation and is recorded in §7 as an open issue for a subsequent release. | Medium |

## 5.4 Software Quality Attributes

| ID | Attribute | Requirement |
| --- | --- | --- |
| NFR-4.1 | Availability | The fleet shall complete all held tasks with the gateway, the dashboard and the access point simultaneously absent. |
| NFR-4.2 | Graceful degradation | Loss of an optional component shall reduce capability by a defined increment (FR-6.9) and shall never halt the fleet. |
| NFR-4.3 | Portability | The planner, traffic model and arbitration logic shall build without source modification for both the ESP32 target and a Linux single-board computer, discharging CON-8. |
| NFR-4.4 | Testability | Every simulation run shall be reproducible from its seed, and every safety decision shall be reconstructable from the logged frames that produced it. |
| NFR-4.5 | Maintainability | Communication, traffic model, planner, auction and arbitration shall be separable modules with defined interfaces, so that one may be replaced without altering the others. |
| NFR-4.6 | Scalability | Adding AMRs shall require no change to the coordination logic and no reconfiguration of existing AMRs beyond zone assignment. |
| NFR-4.7 | Interoperability | Simulated and physical AMRs shall execute identical decision logic, so that simulation results are evidence about the physical system. |

## 5.5 Business Rules

| ID | Rule |
| --- | --- |
| BR-1 | Higher task priority takes precedence at a junction; where priorities are equal the lower robot identifier takes precedence. |
| BR-2 | No task may remain unallocated indefinitely while any AMR is capable of performing it; the aging term enforces this. |
| BR-3 | No AMR may hold more than two queued tasks. |
| BR-4 | An AMR below the battery reserve threshold does not accept new work but does complete work already held. |
| BR-5 | Where bids are near-equal, an idle AMR is preferred to a busy one. |
| BR-6 | A claimed task changes hands only where the improvement exceeds the stability margin. |
| BR-7 | No human-facing component may command an AMR; operators supply work and observe outcomes. |

# 6. Verification and Validation

## 6.1 Verification Methods

| Method | Applied to | Evidence produced |
| --- | --- | --- |
| Inspection | Architectural constraints, packet field widths, absence of prohibited components | Reviewed design record |
| Analysis | Deadlock freedom, memory budget, message-rate arithmetic | Appendix C proof; Appendix D budget |
| Test | All functional requirements | Simulation logs against the case set of §6.2 |
| Demonstration | Degradation behaviour, dashboard, physical demonstrator | Witnessed run, recorded |

## 6.2 Verification Test Cases

The following cases constitute the acceptance test set. Every case has been derived from an operational scenario examined during design; together they exercise every feature in §4.

| TC | Scenario | Expected result | Verifies |
| --- | --- | --- | --- |
| TC-1 | Two AMRs plan routes crossing one junction with overlapping ETA windows | Overlap detected before approach; lower-ranked AMR sheds speed; no stop; no collision | FR-5.1–5.7 |
| TC-2 | Two AMRs approach a single-lane corridor from opposite ends | Yield decision taken at the last passing point before entry; no head-on standoff | FR-5.10, ASM-5 |
| TC-3 | Three AMRs form a cyclic conflict, each waiting on the next | Total order yields a unique maximum; the cycle cannot form; all three clear | FR-5.12, App. C |
| TC-4 | An aisle is blocked mid-route by a dropped pallet | Edge marked impassable; D* Lite repairs locally within 50 ms; blockage gossiped; peers route around | FR-6.1, FR-3.3 |
| TC-5 | An AMR is powered off mid-task | Peers declare loss within 1500 ms; its reservations expire; its task is re-announced and re-won | FR-1.5, FR-6.5 |
| TC-6 | The order gateway is disconnected mid-run | Dashboard goes dark; all held tasks complete; arbitration continues unchanged; new orders resume on reconnection | FR-6.3, NFR-4.1 |
| TC-7 | The Wi-Fi access point is powered off mid-run | Coordination entirely unaffected; telemetry buffers locally | FR-1.7, FR-6.4 |
| TC-8 | The clock beacon is suppressed for more than 60 s | Safety margin widens to 1500 ms; throughput falls; no collision occurs | FR-5.13, FR-7.6 |
| TC-9 | A BID frame is lost, causing two AMRs to compute different winners | Both broadcast CLAIM; lower robot_id retains the task; the other relinquishes within one cycle | FR-4.8 |
| TC-10 | A task is announced while every AMR is at queue capacity or below battery reserve | No bid is received; the task is re-announced; its aging term accrues until an AMR becomes eligible | FR-4.13, FR-4.9 |
| TC-11 | A task at a remote corner competes repeatedly against conveniently located tasks | Aging lowers every bid on the remote task until it is claimed; it is not starved | FR-4.11, BR-2 |
| TC-12 | A busy AMR whose committed route passes the pickup competes against a distant idle AMR | Insertion cost is small; the busy AMR wins; total fleet cost is lower than assigning the idle AMR | FR-4.3 |
| TC-13 | A busy AMR that would have to backtrack competes against a distant idle AMR | Insertion cost is large; the idle AMR wins | FR-4.3 |
| TC-14 | An idle and a busy AMR produce bids differing by less than ε | The idle AMR wins on the idle-preference term | FR-4.3, BR-5 |
| TC-15 | A sequence of marginally better bids arrives for an already-claimed task | No reassignment occurs; the holder retains the task; no thrashing is observed | FR-4.10, BR-6 |
| TC-16 | An AMR already holding two tasks hears a new ANNOUNCE | It does not bid at all; auction traffic does not grow with task backlog | FR-4.9 |
| TC-17 | An AMR's battery falls below reserve during a task | It stops bidding, completes the current task, then routes to a charging node | FR-6.7, FR-4.15 |
| TC-18 | An AMR is introduced to an operating fleet with an uninitialised traffic model | It begins on nominal costs and converges towards the fleet estimate through DIGEST merges | FR-2.7, FR-2.6 |
| TC-19 | Traffic models are deliberately desynchronised across the fleet | AMRs select differing routes; zero collisions are still recorded | FR-2.9, FR-2.10 |
| TC-20 | A sustained burst of tasks routes repeatedly through one corridor | Pheromone accumulates; subsequent routes distribute across alternates; corridor queueing does not form | FR-2.3, FR-3.4 |
| TC-21 | A task is announced in a zone at fleet scale | Only AMRs in that zone and adjacent zones bid; frame count matches the NFR-1.12 budget | FR-9.2, FR-9.3 |
| TC-22 | The full benchmark is executed for both the System and the stop-and-wait baseline over ten seeds | System makespan is at least 20% lower than baseline; System collision count is zero | NFR-2.1, FR-10.4–10.7 |
| TC-23 | A human walks into an aisle ahead of a moving AMR | The AMR stops safely, waits, then repairs its route around the obstruction | FR-6.6, ASM-13 |
| TC-24 | An AMR travels a long route without encountering a marker, then reaches one | Position confidence degrades; speed reduces; confidence is restored on marker decode | FR-7.2, FR-7.3, FR-6.8 |
| TC-25 | The firmware is executed with the maximum graph permitted by ASM-2 | Application SRAM use remains within the NFR-1.10 budget; planning stays within NFR-1.2 | NFR-1.9, NFR-1.10 |
| TC-26 | Two AMRs are placed at the maximum separation at which a collision is geometrically possible | Each receives the other's INTENT; ASM-7 is confirmed for the deployment radio environment | ASM-7, FR-9.4 |
| TC-27 | An AMR with low position confidence approaches a junction | It declines to claim a reservation until confidence is restored | FR-5.14, NFR-2.3 |
| TC-28 | A replayed or malformed frame is injected into the mesh | The frame is discarded with no effect on the reservation table or the traffic model | NFR-3.1–3.3 |

## 6.3 Benchmark Protocol

The success criteria of the problem statement are comparative, and a comparison requires a controlled experiment. The following protocol is mandatory and its output is the evidence for §8.

| Element | Specification |
| --- | --- |
| Map | One warehouse graph containing a single-lane choke corridor and at least two longer alternative routes bypassing it (FR-10.8) |
| Fleet | Not fewer than three AMRs, as mandated; additional configurations at 10 and 100 AMRs for the scalability evidence |
| Task set | A fixed set in which at least 60% of tasks induce a route through the choke corridor, released in waves to force overlap (FR-10.9) |
| Configuration A (control) | Stop-and-wait baseline: round-robin assignment, no intent exchange, halt within radius R and resume when clear |
| Configuration B (System) | Full System as specified in §4 |
| Seeds | Not fewer than ten per configuration, identical seed set across configurations |
| Primary metrics | Makespan; mean per-task completion time; inter-robot collision count |
| Secondary metrics | Total stopped time per AMR; yields granted and received; auction frames per task; route diversity across alternates |
| Reporting | Mean and standard deviation for every metric; a single run is not admissible as evidence |
| Pass condition | Configuration B records zero collisions and a makespan reduction of at least 20% relative to Configuration A |

## 6.4 Acceptance Criteria

| ID | Requirement | Pri. |
| --- | --- | --- |
| AC-1 | All Critical requirements in §4 and §5 shall be verified by their traced test cases. | Critical |
| AC-2 | The benchmark of §6.3 shall record zero inter-robot collisions for the System across all seeds. | Critical |
| AC-3 | The benchmark shall record a makespan reduction of not less than 20% relative to the stop-and-wait baseline. | Critical |
| AC-4 | The fleet shall be demonstrated completing held tasks with the gateway disconnected. | Critical |
| AC-5 | The simulation shall be demonstrated with not fewer than three AMRs, as mandated by the problem statement. | Critical |
| AC-6 | The dashboard shall be demonstrated rendering live position and battery status for the full fleet. | Critical |

# 7. Traceability and Open Issues

## 7.1 Feature to Requirement Traceability

| Feature | Requirements | Verified by |
| --- | --- | --- |
| FE-1 Peer communication | FR-1.1 – FR-1.8, IF-4.1 – IF-4.5 | TC-5, TC-7, TC-26, TC-28 |
| FE-2 Traffic model | FR-2.1 – FR-2.10 | TC-18, TC-19, TC-20 |
| FE-3 Route planning | FR-3.1 – FR-3.9 | TC-4, TC-20, TC-25 |
| FE-4 Task auction | FR-4.1 – FR-4.15 | TC-9 – TC-17 |
| FE-5 Junction arbitration | FR-5.1 – FR-5.14 | TC-1, TC-2, TC-3, TC-8, TC-27 |
| FE-6 Exception handling | FR-6.1 – FR-6.9 | TC-4, TC-5, TC-6, TC-7, TC-17, TC-23 |
| FE-7 Localization and clock | FR-7.1 – FR-7.6 | TC-8, TC-24, TC-27 |
| FE-8 Dashboard | FR-8.1 – FR-8.7, IF-1.1 – IF-1.8 | TC-6, AC-6 |
| FE-9 Zoned scalability | FR-9.1 – FR-9.7 | TC-21, TC-26 |
| FE-10 Metrics and benchmark | FR-10.1 – FR-10.10 | TC-22, AC-2, AC-3 |

## 7.2 Open Issues and Deferred Items

| ID | Issue | Disposition |
| --- | --- | --- |
| OI-1 | Cryptographic peer authentication is not implemented; the fleet is assumed trusted (ASM-15, NFR-3.4) | Deferred to release 2.0; ESP-NOW primary master key is the candidate mechanism |
| OI-2 | Tunable constants α, ε, δ, w₁, w₂ have provisional defaults pending the offline search of FR-10.10 | To be fixed before benchmark baselining |
| OI-3 | Zone boundaries are hand-partitioned; automatic partitioning from the graph is not specified | Acceptable for release 1.0; ASM-16 records the assumption |
| OI-4 | Heterogeneous fleets would break bid comparability (ASM-6) | Out of scope; noted as a limitation in the final presentation |
| OI-5 | Charging dock docking mechanics are out of scope; the System routes to a charging node only | Deferred |
| OI-6 | ASM-7 (colliding robots are always in radio range) requires empirical confirmation in the deployment environment | TC-26 is mandatory before any physical demonstration |

# 8. SIH Problem Statement Requirement Matching

This section maps every clause of Problem Statement 26123 [R4] to the requirements that satisfy it and the cases that verify it. It is the primary compliance artefact of this document.

## 8.1 Requirement Matching Table

| PS clause (verbatim intent) | Satisfied by | Verified by | Status |
| --- | --- | --- | --- |
| Run locally on edge hardware onboard each robot | FE-1 – FE-7 in their entirety; CON-1, CON-8; NFR-4.3 | TC-25, App. D | Satisfied — see §8.2 |
| Decentralized communication: inter-robot messaging sharing position and intent without a central server | FR-1.1 – FR-1.8; IF-4.2; FR-9.4 | TC-5, TC-7, TC-26 | Satisfied |
| Dynamic multi-agent conflict resolution: resolve deadlocks and avoid collisions at narrow intersections and choke points in real time | FR-5.1 – FR-5.14; NFR-1.1; NFR-2.1, NFR-2.2 | TC-1, TC-2, TC-3, TC-27; App. C | Satisfied |
| Task allocation and re-routing: automatically re-assign pickup points or change paths when a robot encounters a blocked aisle | FR-4.1 – FR-4.15; FR-3.3; FR-6.1, FR-6.2, FR-6.5 | TC-4, TC-5, TC-9 – TC-17 | Satisfied |
| Expected: decentralized network stack — a peer-to-peer protocol where robots share localization data locally | §3.4 complete protocol; FR-1.1 – FR-1.8; FR-7.1 – FR-7.5 | TC-26, TC-28 | Satisfied |
| Expected: multi-agent path planning implemented for edge hardware | FR-3.1 – FR-3.9; CON-1; Appendix D | TC-4, TC-20, TC-25 | Satisfied |
| Expected: fleet dashboard — lightweight monitoring UI visualising real-time fleet positions and battery status | FR-8.1 – FR-8.7; IF-1.1 – IF-1.8 | AC-6, TC-6 | Satisfied |
| Expected: a multi-robot simulation featuring at least three AMRs | FR-10.6, FR-10.8; DEP-3; CON-9; AC-5 | AC-5 | Open — build required |
| Success criterion: zero inter-robot collisions | NFR-2.1; FR-10.1; FE-5 entire | TC-22, AC-2 | Open — measurement required |
| Success criterion: minimum 20% reduction in total task completion time versus traditional stop-and-wait when handling overlapping paths | FR-10.4 – FR-10.9; §6.3 benchmark protocol | TC-22, AC-3 | Open — baseline required |

## 8.2 Note on the Edge Hardware Clause

The problem statement names Raspberry Pi or Jetson Nano as illustrative edge hardware, introduced by “e.g.”. This specification targets the ESP32, a platform approximately two orders of magnitude smaller in memory. Three points establish conformance:

- The clause requires that computation run locally on the robot rather than on a server. That requirement is satisfied in full and in a stronger form than the reference platform demands: every operationally necessary decision executes on the robot, and the fleet is complete with all off-robot components absent.

- The mandated deliverable is a multi-robot simulation (CON-9, ASM-14). The platform question therefore concerns the physical demonstrator, not the graded artefact.

- NFR-4.3 requires the planner, traffic model and arbitration logic to build unchanged for a Linux single-board computer. The reference platform therefore remains available without redesign should an evaluator require it.

The deviation is presented as an engineering result rather than a compromise: the reference platform is a Raspberry Pi, and this specification demonstrates the same coordination stack operating within 320 KB of SRAM on a microcontroller costing a small fraction as much, which is the condition under which fleets of one hundred robots become economically deployable.

## 8.3 Residual Risk Against the Success Criteria

Three clauses remain open at the time of baselining, and all three are matters of construction and measurement rather than of design. The critical item is the twenty per cent clause: it cannot be asserted from the architecture and can only be established by executing the protocol of §6.3 against a baseline controller the team must itself build. Until that measurement exists, the criterion is unmet regardless of how well the coordination performs.

# Appendix A — Robot State Model

| State | Entered when | Exited when | Broadcasting |
| --- | --- | --- | --- |
| IDLE | Boot complete, or a task is reported complete | The AMR wins an auction | INTENT with state IDLE |
| BIDDING | An ANNOUNCE for an eligible zone is received | The auction window closes | BID |
| PLANNING | The AMR determines it won the auction | A route is computed, or UNREACHABLE is declared | CLAIM, then INTENT |
| MOVING | A valid route exists | The drop node is reached | INTENT at 5 Hz, RESERVE on claim |
| YIELD | A junction conflict is lost | The conflicting window has passed | INTENT with state YIELD |
| REPLAN | An edge on the committed route becomes impassable | D* Lite returns a repaired route | INTENT with state REPLAN, blockage gossip |
| AT DROP / REPORT | The drop node is reached | COMPLETE is broadcast and the model is updated | COMPLETE, DIGEST |
| CHARGING | Battery falls below reserve and held work is complete | Charge is restored above threshold | INTENT with state CHARGING |
| FAULT | Position confidence lost, hardware fault, or route unrecoverable | Operator or automatic recovery | INTENT with state FAULT |

YIELD and REPLAN are excursions from MOVING and return to it directly. They are normal operating states on a busy floor, not fault states; an AMR that never enters them is an AMR operating alone.

# Appendix B — Junction Arbitration Algorithm

on approaching junction J with planned window [t_in, t_out]:

1. DETECT

conflicts = { r in reservation_table[J] :

overlaps([t_in, t_out], r.window, margin) }

if conflicts is empty -> goto RESERVE

2. PRIORITIZE

me      = (my_priority, my_robot_id)

rival   = max over conflicts of (priority, -robot_id)

// total order: higher priority wins; equal priority -> lower id wins

if me ranks above every rival -> goto RESERVE

else -> goto AVOID

3. RESERVE

broadcast RESERVE(J, t_in, t_out, my_priority, my_robot_id)

enter junction on schedule

4. AVOID

shift = latest conflicting window end + margin - t_in

if speed reduction can absorb 'shift' before the last passing point:

reduce speed; recompute ETAs; rebroadcast INTENT; retry DETECT

else:

mark edge temporarily costly; invoke D* Lite; replan

invariant: no AMR brakes hard as a result of this procedure

# Appendix C — Proof of Deadlock Freedom

Claim. Under the arbitration procedure of Appendix B, no set of AMRs can enter a cyclic wait.

Definitions. Let C be any set of AMRs in mutual conflict over one or more junctions at an instant. Define the ranking key of an AMR as the pair k(r) = (priority(r), −robot_id(r)), ordered lexicographically.

Lemma 1. The relation induced by k is a total order on C. Priority is an integer and robot_id is unique across the fleet by construction (FR-1.2, ASM-17); therefore no two distinct AMRs share a key, and any two keys are comparable. There are no ties and no incomparable pairs.

Lemma 2. Every AMR in C computes the same ranking. Ranking inputs are priority and robot_id, both carried in the broadcast INTENT of every participant (FR-1.2). Arbitration consults no local, learned or randomized state (FR-2.9, FR-5.9) and is deterministic (FR-5.8). Therefore all members of C evaluate the same function over the same inputs and obtain the same result.

Theorem. C contains exactly one AMR that does not yield. Because k is a total order on a finite non-empty set (Lemma 1), C has a unique maximum m. By the PRIORITIZE step, an AMR yields if and only if some rival ranks above it; m has no such rival and therefore proceeds, while every other member of C yields. By Lemma 2 this conclusion is reached identically by every member, so no two AMRs can both proceed and no AMR can be left waiting on a robot that is itself waiting on it.

Corollary. A cyclic wait would require every AMR in the cycle to be waiting on a strictly greater element, which would imply a strictly increasing cycle in a total order. No such cycle exists. The result holds for two AMRs at a junction and for any finite number in a loop.

Scope of the proof. The result depends on ASM-7 (any AMRs able to collide are in mutual radio range) and on FR-5.11 (reservations expire without a release message, so a failed AMR cannot hold a claim indefinitely). Where ASM-7 does not hold, C is not well defined and the guarantee does not apply; TC-26 exists to confirm it empirically.

# Appendix D — Memory Budget

Budget for the maximum graph permitted by ASM-2 (255 nodes, 400 edges). Verifies CON-1, FR-2.8 and NFR-1.10.

| Structure | Sizing | Bytes |
| --- | --- | --- |
| Topological graph | 255 nodes × 4 B + 400 edges × 6 B | 3,420 |
| Learned edge times (EWMA + count) | 400 edges × 3 B | 1,200 |
| Pheromone counters | 400 edges × 1 B | 400 |
| Reservation table | 32 junctions × 4 entries × 6 B | 768 |
| Peer table | 20 peers × 24 B | 480 |
| A* open and closed sets | 255 nodes × 8 B | 2,040 |
| D* Lite g and rhs values | 255 nodes × 8 B | 2,040 |
| Task queue and bid state | 2 tasks × 16 B + auction scratch | 128 |
| Message buffers | 8 frames × 250 B | 2,000 |
| Application total |  | 12,476 |
| FreeRTOS, Wi-Fi and ESP-NOW stacks (typical) |  | ~120,000 |
| Headroom against ~320 KB usable SRAM |  | ~195,000 |

The traffic model proper (learned edge times and pheromone) totals 1,600 bytes at the maximum graph size and 1,000 bytes at a realistic 250-edge warehouse, satisfying FR-2.8 for deployments conforming to typical site scale. An occupancy-grid representation of the same warehouse at 500 × 500 cells would require in excess of two megabytes for the planner alone, which is the analysis underlying the design decision recorded in §2.5.

# Appendix E — Tunable Constants

| Symbol | Meaning | Provisional default | Fixed by |
| --- | --- | --- | --- |
| margin | Junction safety margin | 800 ms | FR-5.3 |
| margin_degraded | Safety margin under clock degradation | 1500 ms | FR-5.13 |
| α | Pheromone weight in edge cost | Offline search | FR-10.10 |
| ε | Idle-preference bid reduction | Offline search | FR-10.10 |
| δ | Task reassignment stability margin | Offline search | FR-10.10 |
| w₁ | Battery penalty weight in bid | Offline search | FR-10.10 |
| w₂ | Task aging weight in bid | Offline search | FR-10.10 |
| queue_cap | Maximum queued tasks per AMR | 2 | FR-4.9, BR-3 |
| peer_timeout | Peer liveness window | 1500 ms | FR-1.5 |
| intent_period | INTENT broadcast period | 200 ms | FR-1.1 |
| digest_period | DIGEST gossip period | 10 s | FR-2.5 |
| auction_window | Bid collection window | 300 ms | FR-4.4 |
| pheromone_decay | Pheromone evaporation period | 2000 ms | FR-2.3 |
| beacon_period | Clock beacon period | 5 s | FR-7.4 |

End of document — ROBOTON-SRS-001 v1.0